Table des matières

I.	Annotation	3

II.	Steps	4

| Révision N° | Modifications | Date | Auteur |
| --- | --- | --- | --- |
| 0 | Création | 31/05/18 | S.HAMMAMI |

- Annotation

| Flange with ears | 300K flange |
| --- | --- |
| PT flange | Stops flange |
| Adjustment rod | Pulsed tube (PT) |
| Nuts Nut n°4 Nut n°3 Nut n°2 Nut n°1 |  |

- Steps

| - Loosen the nut n°1 on each of the three adjustment rods until the flange becomes in contact with the stops |  |
| --- | --- |
| - Loosen the nut n°2 on each of the three adjustment rods of 5mm |  |
| - Take off the three ears of the flange from the stops of the thickness of a sheet of paper |  |
| - Use a multimeter (ohmmeter mode) between the head of the pulsed tube and the 300K flange |  |
| - Squeeze one of the three nuts n°1 and follow the variation of the oscillations of the ohmmeter. |  |
| - Squeeze the three nuts n°2 when the ohmmeter displays “OL” (Overloaded) |  |